Course Code: CCE-332,
Course Title: Peripheral and Interfacing sessional

Group: 01

Comparison with related works:

| Name | Primary Function(s) | Locomotion & Control | Key Features & Innovations |
| --- | --- | --- | --- |
| Farm Droid (our Project) | Real-time monitoring, robotic manipulation, and weeding | Rover, Raspberry Pi 4 (brain), Arduino (controller) | Real time monitoring with weeding, modular robot arm, as well as opencv for image detection. Alongside Gemini is used for LLM integration |
| Modular 4WD Robot [1] | Grass cutting, leaf crushing, and precision seeding | 4WD skid-steer, LiDAR/Camera, EKF state estimation | Features a vertically stacked two-unit architecture with quick-release toolheads. Uses simulated coverage-path algorithms (like zigzag/boustrophedon) for 100% gap-free field coverage. |
| AgriCruiser [2] | Over-the-row precision spraying for weed management | 2-wheel front differential drive with rear casters | Open-source, highly reconfigurable chassis made from T-slot extrusions. Offers adjustable track width (1.42–1.57 m) and high clearance at a low deployment cost of ~$5,000–$6,000. |
| Six-Wheeled Laser Weeder [3] | High-precision laser weed elimination | 6-wheeled, double four-bar linkage suspension | Uses a 3-axis linear actuation mechanism to position a 10W blue diode laser directly over weeds, achieving a 97% hit rate while navigating obstacles up to 15 cm. |
| FarmDroid FD20[4] | Autonomous seeding and preventative mechanical weeding | 4-wheeled, solar-powered, GPS-guided | Operates on a memory-based approach, recording exact GPS coordinates of planted seeds to mechanically weed between and within rows without complex real-time vision. Heavily studied for climate-smart soil management. |
| BoniRob [5][6] | Crop monitoring, targeted fertilization, and weeding | 4-wheel independently steered | Adjusts working width dynamically (0.75m–2.0m) and uses mechanical cylinders to crush weeds as an alternative to herbicide spraying. |
| ROBOTTI 150 D [7] | Multipurpose field operations (evaluated for soil health impact) | Lightweight wheeled vehicle | Used extensively in research to quantify soil compaction. Studies show it causes less subsoil compaction than heavy tractors, though repeated wheeling still alters topsoil porosity. |
| Tertill [8] | Home garden weed control | Solar-powered mini rover | A consumer-grade, waterproof robot that autonomously navigates small plots and uses a simple wire/string trimmer to cut newly sprouted weeds. |

References

References:

[1] Kumar, A., Kamalaksha, S.A., Srividya, R., Zuber, M., Ahmad, K.A., Singh, S. and Nair, V.G., 2026. Modular 4WD agricultural robot for cutting, collection, and precision seeding: design and simulation-based evaluation. Scientific Reports.

[2] Truong, K., 2025. AgriCruiser: An Open Source Agriculture Robot for Over-the-row Navigation. University of California, Los Angeles.

[3] Usama, M., Khan, M.I., Hasan, A., Nadeem, M.S., Iqbal, K.F., Aslam, J., Ali, M.A. and Awan, A.N., 2025. Design of a six wheel suspension and a three-axis linear actuation mechanism for a laser weeding robot. arXiv preprint arXiv:2512.10319.

[4] Bručienė, I., S. Buragienė, and E. Šarauskis. 2025. “Comparative Evaluation of the Effect of Conventional and Robotic Mechanical Weeding on Topsoil Physical Attributes.” Soil and Tillage Research 254: 106739.  https://doi.org/10.1016/j.still.2025.106739

[5] Schaefer, A., Plant Robot “Bonirob”, Available at: https://www.youtube.com/watch?v=utiNto4BeOg, (last accessed 17.06.2026).

[6] Langsenkamp, F., F. Sellmann, M. Kohlbrecher, et al. 2014. “Tube Stamp for Mechanical Intra-Row Indicidual Plant Weed Cover.” Agricultural Engineering International: CIGR Journal: 16–19.

[7] Calleja-Huerta, A., M. Lamandé, O. Green, and L. J. Munkholm. 2023a. “Impacts of Load and Repeated Wheeling From a Lightweight Autonomous Field Robot on the Physical Properties of a Loamy Sand Soil.” Soil and Tillage Research 233: 105791. https:// doi.org/10.1016/j.still.2023.105791

[8] Sanchez, J., and E. R. Gallandt. 2021. “Functionality and Efficacy of Franklin Robotics' Tertill Robotic Weeder.” Weed Technology 35: 166– 170. https://doi.org/ 10.1017/ wet.2020.94