VLAN Configuration Using Cisco Packet Tracer

Basic VLAN • Inter-VLAN Routing • VLAN with DHCP

# 1. Basic VLAN Configuration

## 1.1 Objective

To create VLAN 10 and VLAN 20 on a Cisco 2960 switch, assign PCs to the appropriate VLANs, and verify communication within the same VLAN and isolation between different VLANs.

## 1.2 Network Topology

Cisco 2960 Switch
          +---------------------+
          |                     |
       Fa0/1 Fa0/2 Fa0/3   Fa0/4 Fa0/5 Fa0/6
         |     |     |       |     |     |
        PC1   PC2   PC3     PC4   PC5   PC6
         |     |     |       |     |     |
       VLAN 10: Sales       VLAN 20: Accounts

## 1.3 Devices

1 × Cisco 2960 Switch

6 × PCs

3 PCs for VLAN 10 – Sales

3 PCs for VLAN 20 – Accounts

## 1.4 IP Addressing

| PC | IP Address | Subnet Mask | VLAN |
| --- | --- | --- | --- |
| PC1 | 192.168.10.11 | 255.255.255.0 | 10 |
| PC2 | 192.168.10.12 | 255.255.255.0 | 10 |
| PC3 | 192.168.10.13 | 255.255.255.0 | 10 |
| PC4 | 192.168.20.11 | 255.255.255.0 | 20 |
| PC5 | 192.168.20.12 | 255.255.255.0 | 20 |
| PC6 | 192.168.20.13 | 255.255.255.0 | 20 |

For this basic VLAN exercise, no default gateway is required because communication between VLANs is not being configured.

## 1.5 Create VLANs

Switch> enable
Switch# configure terminal

Switch(config)# vlan 10
Switch(config-vlan)# name SALES
Switch(config-vlan)# exit

Switch(config)# vlan 20
Switch(config-vlan)# name ACCOUNTS
Switch(config-vlan)# exit

## 1.6 Assign Ports to VLAN 10

PC1, PC2 and PC3 are connected to Fa0/1, Fa0/2 and Fa0/3.

Switch(config)# interface range fa0/1 - 3
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 10
Switch(config-if-range)# exit

## 1.7 Assign Ports to VLAN 20

PC4, PC5 and PC6 are connected to Fa0/4, Fa0/5 and Fa0/6.

Switch(config)# interface range fa0/4 - 6
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 20
Switch(config-if-range)# exit

## 1.8 Save the Configuration

Switch(config)# end
Switch# copy running-config startup-config

Press Enter when the switch asks for the destination filename.

## 1.9 Verify VLAN Configuration

Switch# show vlan brief

Expected result:

VLAN Name                             Status    Ports
---- -------------------------------- --------- -------------------------------
1    default                          active    Fa0/7 Fa0/8 ...
10   SALES                            active    Fa0/1 Fa0/2 Fa0/3
20   ACCOUNTS                         active    Fa0/4 Fa0/5 Fa0/6

## 1.10 Test Connectivity

Same VLAN – communication should work:

PC1> ping 192.168.10.12

PC4> ping 192.168.20.12

Different VLAN – communication should fail in this basic Layer 2 setup:

PC1> ping 192.168.20.11

Reason: VLAN 10 and VLAN 20 are separate broadcast domains, and a Layer 2 switch cannot route traffic between them.

## 1.11 Important Switch Commands

| Command | Purpose |
| --- | --- |
| enable | Enter privileged EXEC mode |
| configure terminal | Enter global configuration mode |
| vlan 10 | Create VLAN 10 |
| name SALES | Give VLAN 10 a name |
| interface range fa0/1 - 3 | Select multiple switch ports |
| switchport mode access | Configure ports as access ports |
| switchport access vlan 10 | Assign ports to VLAN 10 |
| show vlan brief | Verify VLANs and assigned ports |
| show running-config | Display current configuration |
| copy running-config startup-config | Save configuration |

# 2. Inter-VLAN Routing Using Router-on-a-Stick

## 2.1 Objective

To allow VLAN 10 (Sales) and VLAN 20 (Accounts) to communicate through a router using Router-on-a-Stick configuration.

## 2.2 Network Topology

Router R1
                       +-----------+
                       |   G0/0    |
                       +-----+-----+
                             |
                         TRUNK LINK
                             |
                       +-----+-----+
                       |    SW1    |
                       +-----------+
                       /           \
                  VLAN 10           VLAN 20
                   SALES            ACCOUNTS
                  /  |  \           / |  \
                PC1 PC2 PC3       PC4 PC5 PC6

## 2.3 Devices

1 × Cisco 2911 Router

1 × Cisco 2960 Switch

6 × PCs

Copper straight-through cables

## 2.4 IP Addressing

| VLAN | Department | Network | Gateway |
| --- | --- | --- | --- |
| 10 | SALES | 192.168.10.0/24 | 192.168.10.1 |
| 20 | ACCOUNTS | 192.168.20.0/24 | 192.168.20.1 |

Router subinterfaces: G0/0.10 = 192.168.10.1 and G0/0.20 = 192.168.20.1.

## 2.5 Configure VLANs on the Switch

Switch> enable
Switch# configure terminal

Switch(config)# vlan 10
Switch(config-vlan)# name SALES
Switch(config-vlan)# exit

Switch(config)# vlan 20
Switch(config-vlan)# name ACCOUNTS
Switch(config-vlan)# exit

## 2.6 Configure PC Access Ports

VLAN 10:
Switch(config)# interface range fa0/1 - 3
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 10
Switch(config-if-range)# exit

VLAN 20:
Switch(config)# interface range fa0/4 - 6
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 20
Switch(config-if-range)# exit

## 2.7 Configure the Trunk Port

Assume the router is connected to Fa0/24.

Switch(config)# interface fa0/24
Switch(config-if)# switchport mode trunk
Switch(config-if)# exit
Switch(config)# end

Switch# show interfaces trunk

## 2.8 Configure the Router

Router> enable
Router# configure terminal

Router(config)# interface gigabitEthernet 0/0
Router(config-if)# no shutdown
Router(config-if)# exit

Router(config)# interface gigabitEthernet 0/0.10
Router(config-subif)# encapsulation dot1Q 10
Router(config-subif)# ip address 192.168.10.1 255.255.255.0
Router(config-subif)# exit

Router(config)# interface gigabitEthernet 0/0.20
Router(config-subif)# encapsulation dot1Q 20
Router(config-subif)# ip address 192.168.20.1 255.255.255.0
Router(config-subif)# exit

Router(config)# end
Router# copy running-config startup-config

## 2.9 Configure PC IP Addresses

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC1 | 192.168.10.11 | 255.255.255.0 | 192.168.10.1 |
| PC2 | 192.168.10.12 | 255.255.255.0 | 192.168.10.1 |
| PC3 | 192.168.10.13 | 255.255.255.0 | 192.168.10.1 |
| PC4 | 192.168.20.11 | 255.255.255.0 | 192.168.20.1 |
| PC5 | 192.168.20.12 | 255.255.255.0 | 192.168.20.1 |
| PC6 | 192.168.20.13 | 255.255.255.0 | 192.168.20.1 |

## 2.10 Test Inter-VLAN Communication

From PC1, ping PC4:

PC1> ping 192.168.20.11

Expected result: Successful. VLAN 10 traffic is routed through R1 to VLAN 20.

## 2.11 Verify the Router

Router# show ip interface brief

Expected:

Interface              IP-Address       Status
GigabitEthernet0/0     unassigned       up
GigabitEthernet0/0.10  192.168.10.1     up
GigabitEthernet0/0.20  192.168.20.1     up

Router# show running-config

## 2.12 Key Concept

Router
              /           \
     192.168.10.1       192.168.20.1
          |                   |
       VLAN 10             VLAN 20
       SALES              ACCOUNTS

The trunk link carries both VLAN 10 and VLAN 20 traffic, while the router subinterfaces perform routing between the two VLANs.

# 3. VLAN with DHCP

## 3.1 Objective

To configure VLAN 10 and VLAN 20 with DHCP so that the router automatically assigns IP addresses, subnet masks, default gateways, and DNS information to PCs in each VLAN.

## 3.2 Topology

R1 Router
                       +-----------+
                       |           |
                       | G0/0      |
                       +-----+-----+
                             |
                         TRUNK LINK
                             |
                       +-----+-----+
                       |    SW1    |
                       +-----------+
                       /           \
                  VLAN 10           VLAN 20
                   SALES            ACCOUNTS
                  /  |  \           / |  \
                PC1 PC2 PC3       PC4 PC5 PC6

## 3.3 VLAN and DHCP Address Plan

| VLAN | Department | Network | Gateway | DHCP Range |
| --- | --- | --- | --- | --- |
| 10 | SALES | 192.168.10.0/24 | 192.168.10.1 | 192.168.10.11–254 |
| 20 | ACCOUNTS | 192.168.20.0/24 | 192.168.20.1 | 192.168.20.11–254 |

The router acts as the DHCP server.

## 3.4 Configure VLANs on the Switch

enable
configure terminal

vlan 10
name SALES
exit

vlan 20
name ACCOUNTS
exit

## 3.5 Assign PC Ports

VLAN 10:
interface range fa0/1 - 3
switchport mode access
switchport access vlan 10
exit

VLAN 20:
interface range fa0/4 - 6
switchport mode access
switchport access vlan 20
exit

## 3.6 Configure the Trunk Port

interface fa0/24
switchport mode trunk
exit

end

show vlan brief
show interfaces trunk

## 3.7 Configure Router Subinterfaces

enable
configure terminal

interface gigabitEthernet 0/0
no shutdown
exit

interface gigabitEthernet 0/0.10
encapsulation dot1Q 10
ip address 192.168.10.1 255.255.255.0
exit

interface gigabitEthernet 0/0.20
encapsulation dot1Q 20
ip address 192.168.20.1 255.255.255.0
exit

## 3.8 Configure DHCP for VLAN 10

ip dhcp excluded-address 192.168.10.1 192.168.10.10

ip dhcp pool SALES
network 192.168.10.0 255.255.255.0
default-router 192.168.10.1
dns-server 8.8.8.8
exit

PCs in VLAN 10 can receive addresses such as 192.168.10.11, 192.168.10.12, 192.168.10.13, and so on.

## 3.9 Configure DHCP for VLAN 20

ip dhcp excluded-address 192.168.20.1 192.168.20.10

ip dhcp pool ACCOUNTS
network 192.168.20.0 255.255.255.0
default-router 192.168.20.1
dns-server 8.8.8.8
exit

PCs in VLAN 20 can receive addresses such as 192.168.20.11, 192.168.20.12, 192.168.20.13, and so on.

## 3.10 Save Router Configuration

end
copy running-config startup-config

## 3.11 Configure PCs for DHCP

On each PC, select Desktop → IP Configuration → DHCP. Repeat for PC1 through PC6.

Example PC1:

IP Address:       192.168.10.11
Subnet Mask:      255.255.255.0
Default Gateway: 192.168.10.1

Example PC4:

IP Address:       192.168.20.11
Subnet Mask:      255.255.255.0
Default Gateway: 192.168.20.1

The exact address assigned to each PC can vary depending on DHCP lease order.

## 3.12 Verify DHCP

Router# show ip dhcp binding
Router# show ip dhcp pool

The DHCP binding table should display the IP addresses leased to client PCs.

## 3.13 Test Connectivity

Test within VLAN 10:

PC1> ping 192.168.10.12

Expected: Successful.

Test within VLAN 20:

PC4> ping 192.168.20.12

Expected: Successful.

Test between VLANs:

PC1> ping 192.168.20.11

Expected: Successful because the router performs Inter-VLAN Routing.

## 3.14 Complete Router Configuration for VLAN + DHCP

enable
configure terminal

interface gigabitEthernet 0/0
no shutdown
exit

interface gigabitEthernet 0/0.10
encapsulation dot1Q 10
ip address 192.168.10.1 255.255.255.0
exit

interface gigabitEthernet 0/0.20
encapsulation dot1Q 20
ip address 192.168.20.1 255.255.255.0
exit

ip dhcp excluded-address 192.168.10.1 192.168.10.10
ip dhcp excluded-address 192.168.20.1 192.168.20.10

ip dhcp pool SALES
network 192.168.10.0 255.255.255.0
default-router 192.168.10.1
dns-server 8.8.8.8
exit

ip dhcp pool ACCOUNTS
network 192.168.20.0 255.255.255.0
default-router 192.168.20.1
dns-server 8.8.8.8
exit

end
copy running-config startup-config

## 3.15 Overall Working Principle

PC
 ↓
Access Port
 ↓
VLAN
 ↓
Trunk
 ↓
Router Subinterface
 ↓
DHCP Pool
 ↓
Automatic IP Address

# 4. Practical Summary

| Lab | Main Topics | Expected Result |
| --- | --- | --- |
| Basic VLAN | VLAN creation, access ports, verification | Same-VLAN communication; different VLANs isolated |
| Inter-VLAN Routing | VLAN + trunk + Router-on-a-Stick | VLAN 10 and VLAN 20 can communicate |
| VLAN with DHCP | VLAN + trunk + routing + DHCP | PCs automatically receive IP configuration and can communicate across VLANs |

Note: Interface names can differ depending on the router/switch model selected in Packet Tracer. If your router uses a different GigabitEthernet interface, use the corresponding interface name.