| Interface | Host Address | Subnet Mask | Default Gateway | Network Address |
| --- | --- | --- | --- | --- |
| Distribution Router 1 (CSE) |  |  |  |  |
| GE0/0 | 10.28.88.2 | 255.255.255.248 |  | 10.28.88.0/29 |
| GE2/0.10 | 10.28.32.1 | 255.255.248.0 |  | 10.28.32.0/21 |
| GE2/0.20 | 10.28.64.1 | 255.255.252.0 |  | 10.28.64.0/22 |
| GE2/0.30 | 10.28.80.1 | 255.255.254.0 |  | 10.28.80.0/23 |
| Distribution Router 2 (BBA) |  |  |  |  |
| GE0/0 | 10.28.88.10 | 255.255.255.248 |  | 10.28.88.8/29 |
| GE2/0.10 | 10.28.40.1 | 255.255.248.0 |  | 10.28.40.0/21 |
| GE2/0.20 | 10.28.68.1 | 255.255.252.0 |  | 10.28.68.0/22 |
| GE2/0.30 | 10.28.82.1 | 255.255.254.0 |  | 10.28.82.0/23 |
| Distribution Router 3 (NFS) |  |  |  |  |
| GE0/0 | 10.28.88.18 | 255.255.255.248 |  | 10.28.88.16/29 |
| GE2/0.10 | 10.28.48.1 | 255.255.248.0 |  | 10.28.48.0/21 |
| GE2/0.20 | 10.28.72.1 | 255.255.252.0 |  | 10.28.72.0/22 |
| GE2/0.30 | 10.28.84.1 | 255.255.254.0 |  | 10.28.84.0/23 |
| Core Router (Server Room) |  |  |  |  |
| GE0/3 | 103.133.254.2 | 255.255.255.0 |  |  |
| GE0/0 | 10.28.88.1 | 255.255.255.248 |  | 10.28.88.0/29 |
| GE0/1 | 10.28.88.9 | 255.255.255.248 |  | 10.28.88.8/29 |
| GE0/2 | 10.28.88.17 | 255.255.255.248 |  | 10.28.88.16/29 |
| Optional |  |  |  |  |
| Remote Core Router |  |  |  |  |
| GE0/0 | 103.133.254.1 | 255.255.255.0 |  |  |
| DNS Server | 103.133.254.2 | 255.255.255.0 | 103.133.254.1 |  |
| PC 0 | 103.133.254.3 | 255.255.255.0 | 103.133.254.1 |  |

## Addressing Table 1

## IP DHCP Pools

| Pool Identity | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Distribution Router 1 |  |  |  |
| VLAN10 | 10.28.32.0 | 255.255.248.0 | 10.28.32.1 |
| EXCLUDE ADDRESS | 10.28.32.1 |  |  |
| EXCLUDE ADDRESS | 10.28.39.255 |  |  |
| VLAN20 | 10.28.64.0 | 255.255.252.0 | 10.28.64.1 |
| EXCLUDE ADDRESS | 10.28.64.1 |  |  |
| EXCLUDE ADDRESS | 10.28.67.255 |  |  |
| VLAN30 | 10.28.80.0 | 255.255.254.0 | 10.28.80.1 |
| EXCLUDE ADDRESS | 10.28.80.1 |  |  |
| EXCLUDE ADDRESS | 10.28.81.255 |  |  |
| Distribution Router 2 |  |  |  |
| VLAN10 | 10.28.40.0 | 255.255.248.0 | 10.28.40.1 |
| EXCLUDE ADDRESS | 10.28.40.1 |  |  |
| EXCLUDE ADDRESS | 10.28.47.255 |  |  |
| VLAN20 | 10.28.68.0 | 255.255.252.0 | 10.28.68.1 |
| EXCLUDE ADDRESS | 10.28.68.1 |  |  |
| EXCLUDE ADDRESS | 10.28.71.255 |  |  |
| VLAN30 | 10.28.82.0 | 255.255.254.0 | 10.28.82.1 |
| EXCLUDE ADDRESS | 10.28.82.1 |  |  |
| EXCLUDE ADDRESS | 10.28.83.255 |  |  |
| Distribution Router 3 |  |  |  |
| VLAN10 | 10.28.48.0 | 255.255.248.0 | 10.28.48.1 |
| EXCLUDE ADDRESS | 10.28.48.1 |  |  |
| EXCLUDE ADDRESS | 10.28.55.255 |  |  |
| VLAN20 | 10.28.72.0 | 255.255.252.0 | 10.28.72.1 |
| EXCLUDE ADDRESS | 10.28.72.1 |  |  |
| EXCLUDE ADDRESS | 10.28.75.255 |  |  |
| VLAN30 | 10.28.84.0 | 255.255.254.0 | 10.28.84.1 |
| EXCLUDE ADDRESS | 10.28.84.1 |  |  |
| EXCLUDE ADDRESS | 10.28.85.255 |  |  |

## Distribution Switch 1, 2, 3 VLANs

| VLAN ID | VLAN Name | Port Range | VLAN Mode |
| --- | --- | --- | --- |
| 10 | Student | Port FE0/1-FE0/6 | Access |
| 20 | Teacher | Port FE0/7-FE0/12 | Access |
| 30 | Academic | Port FE0/13-FE0/18 | Access |
| 1 | Default | GE0/1, GE0/2 | Trunk |

Small form-factor pluggable, or SFP

| GE2/0.10 | 10.28.32.1 | 255.255.248.0 |
| --- | --- | --- |
| GE2/0.20 | 10.28.64.1 | 255.255.252.0 |
| GE2/0.30 | 10.28.80.1 | 255.255.254.0 |

# Basic Campus Network Design

Let’s assume, we Have 3 buildings on our campus.

1. student CSE/bba/nfs , where around 2000 students .

2. Teachers CSE/BBA/NFS, where approximately 1000.

3. Academic building, where nearly 500 staffs work.

Our objective is to connect all of their devices through a network and provide internet access to everyone. Therefore, we would like to create 3 individual networks for three buildings.

## 3500----Total

2^12=4096

## 32

20 bit

## Main Block 10.28.0.0/19

## Block Core 10.28.88.0/27

Sub=n+log2 N/Ni=27+ log2 (32/8) =27+log2 4=27+2=29

## Core to dis 1:   Block Core 10.28.88.0-7/29

## Core to dis 2: Block Core 10.28.88.8-15/29

## Core to dis 3:  Block Core 10.28.88.16-23/29

Point to network 2 bit   4  ip address

## Main Block 10.28.0.0/19

2^13=8192

N=n+log2 (N/Ni)=19+Log2 (8192/2048)=19+ Log24=19+2=21   Student/21

N=n+log2 (N/Ni)=19+Log2 (8192/1024)=19+ Log28=19+3=22   teacher  /22

N=n+log2 (N/Ni)=19+Log2 (8192/512)=19+ Log216=19+4=23   Academic/.23

Student CSE

2000

2048

2^11=11 bit host  21 bit net ==2048

## Block distribution 10.28.32.0/21=n

## Block distribution 10.28.32.1/21]

------

## Block distribution 10.28.32.255/21

## Block distribution 10.28.33.0-255/21 =256

## Block distribution 10.28.34.0-255/21 =256

## Block distribution 10.28.35.0-255/21 =256

## Block distribution 10.28.36.0-255/21 =256

## Block distribution 10.28.37.0-255/21 =256

## Block distribution 10.28.38.0-255/21 =256

## Block distribution 10.28.39.0-10.28.39.255/21 =256

## ---2048

Student BBA

## Block distribution 10.28.40.0/21

## Block distribution 10.28.40.1/21]

------

## Block distribution 10.28.40.255/21

## Block distribution 10.28.41.0-255/21 =256

## Block distribution 10.28.42.0-255/21 =256

## Block distribution 10.28.43.0-255/21 =256

## Block distribution 10.28.44.0-255/21 =256

## Block distribution 10.28.45.0-255/21 =256

## Block distribution 10.28.46.0-255/21 =256

## Block distribution 10.28.47.0-10.28.47.255/21 =256

## ---2048

Student NFS

## Block distribution 10.28.48.0/21

## Block distribution 10.28.48.1/21]

------

## Block distribution 10.28.48.255/21

## Block distribution 10.28.49.0-255/21 =256

## Block distribution 10.28.50.0-255/21 =256

## Block distribution 10.28.51.0-255/21 =256

## Block distribution 10.28.52.0-255/21 =256

## Block distribution 10.28.53.0-255/21 =256

## Block distribution 10.28.54.0-255/21 =256

## Block distribution 10.28.55.0-10.28.55.255/21 =256

## ---2048

Student 2000

2048

2^11=2048 user

21 bit Network and 11 bit Host

Total ranges 10.28.32.0 /21---10.28.39.255

Subnet mask: 255.255.248.0

Teacher 1000

1024

2^10=1024

Net id 22 and Host 10 bit  /22

10.28.64.0 =00001010. 00011100 .01000000.00000000

255.255.248.0

10.28.64.1

00001010. 01000000.0

10.28.64.0 /22-

CSE teacher

10.28.64.0 /22-256

10.28.65.0 /22-256

10.28.66.0 /22-256

10.28.67.0-10.28.67.255 /22-256

BBA teacher

10.28.68.0 /22-256

10.28.69.0 /22-256

10.28.70.0 /22-256

10.28.71.0-10.28.71.255 /22-256

NFS teacher

10.28.72.0 /22-256

10.28.73.0 /22-256

10.28.74.0 /22-256

10.28.75.0-10.28.75.255 /22-256

Admin 500   =512    9 bit  host  23 but netid

/23

CSE Admin

10.28.80.0 /23-256

10.28.81.0-10.28.81.255 /23-256

512

BBA Admin

10.28.82.0 /23-256

10.28.83.0-10.28.83.255 /23-256

512

NFS Admin

10.28.84.0 /23-256

10.28.85.0-10.28.85.255 /23-256

512

10.28.32.0/21

10.28.0000

Core router Configuration

router ospf 1

router-id 1.2.3.0

log-adjacency-changes

redistribute bgp 65002 subnets

network 10.28.88.0 0.0.0.7 area 0

network 10.28.88.8 0.0.0.7 area 1

network 10.28.88.16 0.0.0.7 area 2

!

router bgp 65002

bgp log-neighbor-changes

no synchronization

neighbor 103.133.254.1 remote-as 65001

network 103.133.254.0 mask 255.255.255.0

redistribute ospf 1

Distribution Router 01

interface GigabitEthernet0/0

ip address 10.28.88.2 255.255.255.248

OSPF Configuration

router ospf 1

router-id 1.1.1.1

log-adjacency-changes

network 10.28.88.0 0.0.0.7 area 0

network 10.28.32.0 0.0.7.255 area 0

network 10.28.64.0 0.0.3.255 area 0

network 10.28.80.0 0.0.1.255 area 0

!

router rip

!

ip classless

VLAN Protocal

Router#conf

Router#configure ter

Router#configure terminal

Router(config)#interface

Router(config)#interface gig

Router(config)#interface gigabitEthernet 1/0.10

Router(config-subif)#des

Router(config-subif)#description vlan_10subinterface

Router(config-subif)#enc

Router(config-subif)#encapsulation dot1Q 10

Router(config-subif)#ip add

Router(config-subif)#ip address 10.28.32.1 255.255.224.0

Router(config)#ip dhcp pool vlan10

Router(dhcp-config)#net

Router(dhcp-config)#network 10.28.32.0 255.255.224.0

Router(dhcp-config)#def

Router(dhcp-config)#default-router 10.28.32.1

Router(config)#ip dhcp exclud

Router(config)#ip dhcp excluded-address 10.28.32.1

Router(config)#ip dhcp excluded-address 10.28.63.255