ADVANCED NETWORKING LABORATORY PROBLEM

Campus Network Design, Simulation and Physical Hardware Implementation

| Course | Computer Networks / Network Routing and Switching |
| --- | --- |
| Laboratory | Advanced Campus Network Configuration |
| Total Marks | 30 |
| Duration | 3–4 Hours |
| Software | GNS3 or Cisco Packet Tracer |
| Hardware | MikroTik/Cisco Router, Cisco SG350 Managed Switch, PC/Laptop, Access Point, CAT-6 and Fiber/SFP |

# 1. Problem Statement

A university wants to establish an integrated campus network connecting three major academic divisions: CSE, BBA and NFS. The campus network consists of a central Core Router, three Distribution Routers, three Cisco SG350 managed switches, wireless access points, client computers, and an Internet/ISP connection. The network must provide separate logical networks for Student, Teacher and Academic/Staff users.

Students are required to first design and implement the complete network in GNS3 or Cisco Packet Tracer, verify that it works correctly, save the simulation/project file, and then reproduce the same network using the actual hardware available in the networking laboratory.

# 2. Required Network Topology

Construct the following logical topology:

INTERNET / ISP
                              |
                         Core Router
                    /          |          \
                 Fiber       Fiber       Fiber
                  /            |            \
             DR-CSE         DR-BBA        DR-NFS
             Router          Router         Router
                |              |              |
              CAT6           CAT6           CAT6
                |              |              |
             SG350-1         SG350-2        SG350-3
             Switch          Switch         Switch
           /    |    \      /   |   \      /   |   \
        VLAN10 VLAN20 VLAN30 VLAN10 ... VLAN10 VLAN20 VLAN30

# 3. Network Addressing

Distribution Router 1 — CSE

| Interface | Address | Mask | Network |
| --- | --- | --- | --- |
| WAN/Core | 10.28.88.2 | 255.255.255.248 | 10.28.88.0/29 |
| VLAN 10 Student | 10.28.32.1 | 255.255.248.0 | 10.28.32.0/21 |
| VLAN 20 Teacher | 10.28.64.1 | 255.255.252.0 | 10.28.64.0/22 |
| VLAN 30 Academic | 10.28.80.1 | 255.255.254.0 | 10.28.80.0/23 |

Distribution Router 2 — BBA

| Interface | Address | Mask | Network |
| --- | --- | --- | --- |
| WAN/Core | 10.28.88.10 | 255.255.255.248 | 10.28.88.8/29 |
| VLAN 10 Student | 10.28.40.1 | 255.255.248.0 | 10.28.40.0/21 |
| VLAN 20 Teacher | 10.28.68.1 | 255.255.252.0 | 10.28.68.0/22 |
| VLAN 30 Academic | 10.28.82.1 | 255.255.254.0 | 10.28.82.0/23 |

Distribution Router 3 — NFS

| Interface | Address | Mask | Network |
| --- | --- | --- | --- |
| WAN/Core | 10.28.88.18 | 255.255.255.248 | 10.28.88.16/29 |
| VLAN 10 Student | 10.28.48.1 | 255.255.248.0 | 10.28.48.0/21 |
| VLAN 20 Teacher | 10.28.72.1 | 255.255.252.0 | 10.28.72.0/22 |
| VLAN 30 Academic | 10.28.84.1 | 255.255.254.0 | 10.28.84.0/23 |

Core Router — Server Room

| Interface | IP Address | Mask |
| --- | --- | --- |
| Core → CSE | 10.28.88.1 | 255.255.255.248 |
| Core → BBA | 10.28.88.9 | 255.255.255.248 |
| Core → NFS | 10.28.88.17 | 255.255.255.248 |
| Core → ISP | 103.133.254.2 | 255.255.255.0 |

# 4. VLAN Configuration

| VLAN ID | VLAN Name | Port Range | Mode |
| --- | --- | --- | --- |
| 10 | Student | FE0/1–FE0/6 | Access |
| 20 | Teacher | FE0/7–FE0/12 | Access |
| 30 | Academic | FE0/13–FE0/18 | Access |
| 1 | Default | Uplink ports | Trunk |

Create VLAN 10 (Student), VLAN 20 (Teacher), and VLAN 30 (Academic).

Assign the appropriate access ports to each VLAN.

Configure the router/switch uplink as a trunk.

Allow VLANs 10, 20 and 30 across the trunk.

Verify VLAN membership and trunk operation.

# 5. Inter-VLAN Routing

Configure Router-on-a-Stick on each Distribution Router using 802.1Q subinterfaces. Use VLAN 10, VLAN 20 and VLAN 30 subinterfaces with the gateway addresses specified in Section 3.

Students must demonstrate communication between Student, Teacher and Academic VLANs within each distribution network.

# 6. DHCP Configuration

| Site | VLAN 10 Student | VLAN 20 Teacher | VLAN 30 Academic |
| --- | --- | --- | --- |
| CSE | 10.28.32.0/21 | 10.28.64.0/22 | 10.28.80.0/23 |
| BBA | 10.28.40.0/21 | 10.28.68.0/22 | 10.28.82.0/23 |
| NFS | 10.28.48.0/21 | 10.28.72.0/22 | 10.28.84.0/23 |

Each DHCP client must automatically obtain an IP address, subnet mask, default gateway and DNS server.

Exclude the gateway and appropriate reserved addresses from every DHCP pool.

# 7. OSPF Configuration

| Router | OSPF Router ID | Area |
| --- | --- | --- |
| Core Router | 1.2.3.0 | 0/1/2 |
| Distribution Router 1 (CSE) | 1.1.1.1 | 0 |
| Distribution Router 2 (BBA) | 2.2.2.2 | 1 |
| Distribution Router 3 (NFS) | 3.3.3.3 | 2 |

Configure OSPF between the Core Router and all three Distribution Routers. Advertise all campus VLAN networks and the appropriate /29 core links.

Verify OSPF using show ip ospf neighbor, show ip route, show ip protocols, or the equivalent MikroTik RouterOS commands.

# 8. Core Router and ISP Connectivity

Configure the Core Router using the following point-to-point links:

| Link | Core Address | Remote Address | Network |
| --- | --- | --- | --- |
| Core–CSE | 10.28.88.1 | 10.28.88.2 | 10.28.88.0/29 |
| Core–BBA | 10.28.88.9 | 10.28.88.10 | 10.28.88.8/29 |
| Core–NFS | 10.28.88.17 | 10.28.88.18 | 10.28.88.16/29 |

For the ISP portion, use 103.133.254.0/24. The supplied design uses Core Router AS 65002 and ISP/Remote Router AS 65001.

Configure the external routing relationship and demonstrate reachability to the ISP/external network.

# 9. DNS and Server Network

| Device | IP Address | Mask | Gateway |
| --- | --- | --- | --- |
| External/Server Network | 103.133.254.0/24 | 255.255.255.0 | — |
| Remote/ISP Router | 103.133.254.1 | 255.255.255.0 | — |
| DNS Server | 103.133.254.2 | 255.255.255.0 | 103.133.254.1 |
| External PC | 103.133.254.3 | 255.255.255.0 | 103.133.254.1 |

Test campus-client connectivity to the DNS server, external router and ISP network.

# 10. Wireless Network

Connect at least one Access Point to each Distribution Switch. Configure wireless access for the appropriate campus users and demonstrate connectivity from a laptop, smartphone or other wireless client.

# 11. Network Verification

PC → Local VLAN gateway

VLAN 10 → VLAN 20

VLAN 20 → VLAN 30

VLAN 10 → VLAN 30

CSE → BBA

CSE → NFS

BBA → NFS

CSE/BBA/NFS → Core Router

CSE/BBA/NFS → ISP network

Recommended verification commands:

show ip interface brief

show ip route

show ip ospf neighbor

show ip protocols

show vlan brief

show interfaces trunk

show running-config

ping

traceroute / tracert

# 12. Troubleshooting Task

The instructor will intentionally introduce at least two network faults. Students must identify the fault, explain the cause, correct the configuration and demonstrate restored connectivity.

Wrong VLAN assignment

Incorrect subnet mask or default gateway

Disabled interface

Incorrect trunk configuration

Incorrect VLAN ID

Incorrect or missing OSPF network statement

Incorrect BGP neighbor

DHCP configuration error

Incorrect cable/fiber connection

# 13. Physical Hardware Implementation

After successful simulation, reproduce the same topology using the actual laboratory hardware. The physical implementation should include the Core Router, three Distribution Routers, three SG350 managed switches, Access Points, PCs/laptops, fiber/SFP uplinks and CAT-6 connections.

Physically connect the Core Router and three Distribution Routers.

Establish the fiber/SFP uplinks.

Connect each Distribution Router to its SG350 switch using CAT-6.

Configure VLANs 10, 20 and 30.

Configure trunk ports and Router-on-a-Stick.

Configure DHCP and OSPF.

Configure BGP/ISP connectivity as required.

Connect PCs and wireless clients.

Verify end-to-end communication.

# 14. Simulation Requirement

Option A — GNS3: Submit the complete GNS3 project with all required configuration files.

Option B — Cisco Packet Tracer: Submit the .pkt file. Equivalent Cisco devices may be used, but the logical topology, VLAN IDs, IP addressing and routing objectives must be maintained.

Do not change the supplied IP plan merely because a different simulation platform is used.

# 15. Required Submission

GNS3 project or Cisco Packet Tracer .pkt file

Complete network topology diagram

IP addressing table

VLAN table

DHCP configuration/table

Core Router configuration

Distribution Router 1 configuration

Distribution Router 2 configuration

Distribution Router 3 configuration

SG350-1, SG350-2 and SG350-3 configurations

OSPF configuration and verification

BGP/ISP configuration and verification

DNS/Server connectivity results

Wireless configuration and verification

Screenshots of successful simulation

Photographs/screenshots of physical hardware implementation

Troubleshooting results

Short laboratory report

# 17. Final Student Instruction

Design the given university campus network in GNS3 or Cisco Packet Tracer using the supplied topology and IP addressing scheme. Configure VLANs 10, 20 and 30 for Student, Teacher and Academic users; implement inter-VLAN routing, DHCP, OSPF, BGP/ISP connectivity, DNS and wireless access. Verify complete end-to-end connectivity and troubleshoot intentionally introduced faults. After successful simulation, reproduce the topology and configuration using the actual routers, SG350 managed switches, access points, PCs, fiber/SFP and CAT-6 hardware available in the networking laboratory. Submit the simulation file, configuration files, verification evidence and physical implementation report.

# Instructor Note

The source materials contain a few apparent inconsistencies in interface naming and some external/server addressing entries. For the laboratory, the instructor should designate one authoritative addressing table before students begin. This problem follows the main topology and addressing plan supplied with the laboratory materials rather than silently replacing it.