Distribution Router RB 3011 series

Step 1:

Default IP: 192.168.88.1 in browser

User:

Password:

Reset Router from GUI --- system – reset configuration or hardware button press

Step 2: Select interface list and change wan to SFP 1

Step 3: Go to Interface  and rename SFP1 to wan and eth 1 to lan

Step 4: Go to VLAN and create 3 VLAN  as student, teacher and academic as 10, 20, 30

Step 5: Assign IP address go to ip address IP then add address

| Distribution Router 3 | Ip address | subnet mask | Interface | Network address |
| --- | --- | --- | --- | --- |
| GE0/0 | 10.28.88.18/29 | 255.255.255.248 | Wan | 10.28.88.16/29 |
| GE2/0.10 | 10.28.48.1/21 | 255.255.248.0 | Lan | 10.28.48.0/21 |
| GE2/0.20 | 10.28.72.1/22 | 255.255.252.0 | Lan | 10.28.72.0/22 |
| GE2/0.30 | 10.28.84.1/23 | 255.255.254.0 | lan | 10.28.84.0/23 |

Output

Step 6 Routing: OSPF configuration ---go to routing then select ospf

Select instances –create instances

Create areas : Select areas

Output

Step 7: add network in OSPF

Network address

10.28.88.16/29

10.28.48.0/21

10.28.72.0/22

10.28.84.0/23

Output

Step 8 : DHCP configuration  select IP then DHCP server

Check Routes and neighbor