Distribution 1 Router

/interface ethernet

set [ find default-name=ether3 ] name=lan

set [ find default-name=ether2 ] name=wan

/interface vlan

add interface=lan name=academic vlan-id=30

add interface=lan name=student vlan-id=10

add interface=lan name=teacher vlan-id=20

/ip address

add address=10.28.88.2/29 interface=wan network=10.28.88.0

add address=10.28.32.1/21 interface=student network=10.28.32.0

add address=10.28.64.1/22 interface=teacher network=10.28.64.0

add address=10.28.80.1/23 interface=academic network=10.28.80.0

DHCP command

/ip pool

add name=dhcp_pool0 ranges=10.28.32.2-10.28.39.254

add name=dhcp_pool1 ranges=10.28.64.2-10.28.67.254

add name=dhcp_pool2 ranges=10.28.32.2-10.28.39.254

add name=dhcp_pool3 ranges=10.28.80.2-10.28.81.254

/ip dhcp-server

add address-pool=dhcp_pool1 disabled=no interface=teacher name=dhcp2

add address-pool=dhcp_pool2 disabled=no interface=student name=dhcp1

add address-pool=dhcp_pool3 disabled=no interface=academic name=dhcp3

/ip dhcp-server network

add address=10.28.32.0/21 dns-server=103.133.254.2,8.8.8.8 gateway=10.28.32.1

add address=10.28.64.0/22 dns-server=103.133.254.2,8.8.8.8 gateway=10.28.64.1

add address=10.28.80.0/23 dns-server=103.133.254.2,8.8.8.8 gateway=10.28.80.1

OSPF

/routing ospf instance

add name="OSPF 1" router-id=1.1.1.1

/routing ospf area

add area-id=1.1.1.1 instance="OSPF 1" name=area1

/routing ospf network

add area=area1 network=10.28.88.0/29

add area=area1 network=10.28.32.0/21

add area=area1 network=10.28.64.0/22

add area=area1 network=10.28.80.0/23