Addressing table

| Interface | Host Address | Subnet Mask | Default Gateway | Network Address |
| --- | --- | --- | --- | --- |
| Distribution Router 1 (CSE) |  |  |  |  |
| GE0/2 | 10.28.88.2/29 | 255.255.255.248 |  | 10.28.88.0/29 |
| GE2/0.10 =student | 10.28.32.1/21 | 255.255.248.0 | 2048 | 10.28.32.0/21 |
| GE2/0.20 | 10.28.64.1/22 | 255.255.252.0 |  | 10.28.64.0/22 |
| GE2/0.30 | 10.28.80.1/23 | 255.255.254.0 |  | 10.28.80.0/23 |
| Distribution Router 2 (BBA) |  |  |  |  |
| SF1 | 10.28.88.10/29 | 255.255.255.248 |  | 10.28.88.8/29 |
| GE2/0.10 | 10.28.40.1/21 | 255.255.248.0 |  | 10.28.40.0/21 |
| GE2/0.20 | 10.28.68.1/22 | 255.255.252.0 |  | 10.28.68.0/22 |
| GE2/0.30 | 10.28.82.1/23 | 255.255.254.0 |  | 10.28.82.0/23 |
| Distribution Router 3 (NFS) |  |  |  |  |
| SFP1 | 10.28.88.18/29 | 255.255.255.248 |  | 10.28.88.16/29 |
| GE2/0.10 | 10.28.48.1/21 | 255.255.248.0 |  | 10.28.48.0/ |
| GE2/0.20 | 10.28.72.1/22 | 255.255.252.0 |  | 10.28.72.0/22 |
| GE2/0.30 | 10.28.84.1/23 | 255.255.254.0 |  | 10.28.84.0/23 |
| Core Router (Server Room) |  |  |  |  |
| GE0/3 | 103.133.254.2 | 255.255.255.0 |  |  |
| SFP2 | 10.28.88.1 | 255.255.255.248 |  | 10.28.88.0/29 |
| SF3 | 10.28.88.9 | 255.255.255.248 |  | 10.28.88.8/29 |
| SF4 | 10.28.88.17 | 255.255.255.248 |  | 10.28.88.16/29 |
| Optional |  |  |  |  |
| Remote Core Router |  |  |  |  |
| GE0/0 | 103.133.254.1 | 255.255.255.0 |  |  |
| DNS Server | 103.133.254.2 | 255.255.255.0 | 103.133.254.1 |  |
| PC 0 | 103.133.254.3 | 255.255.255.0 | 103.133.254.1 |  |

128 64 32 16 8  4 2 1

1      1    1  1   1  1 1 1

## Distribution Switch 1, 2, 3 VLANs

| VLAN ID | VLAN Name | Port Range | VLAN Mode |
| --- | --- | --- | --- |
| 10 | Student | Port FE0/1-FE0/6 | Access |
| 20 | Teacher | Port FE0/7-FE0/12 | Access |
| 30 | Academic | Port FE0/13-FE0/18 | Access |
| 1 | Default | GE0/25, GE0/6 GE0/49, GE0/50 | Trunk |

Small form-factor pluggable, or SFP

1

## DHCP Pools

| Pool Identity | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Distribution Router 1 |  |  |  |
| VLAN10 | 10.28.32.0 | 255.255.248.0 | 10.28.32.1 |
| EXCLUDE ADDRESS | 10.28.32.1 |  |  |
| EXCLUDE ADDRESS | 10.28.39.255 |  |  |
| VLAN20 | 10.28.64.0 | 255.255.252.0 | 10.28.64.1 |
| EXCLUDE ADDRESS | 10.28.64.1 |  |  |
| EXCLUDE ADDRESS | 10.28.67.255 |  |  |
| VLAN30 | 10.28.80.0 | 255.255.254.0 | 10.28.80.1 |
| EXCLUDE ADDRESS | 10.28.80.1 |  |  |
| EXCLUDE ADDRESS | 10.28.81.255 |  |  |
| Distribution Router 2 |  |  |  |
| VLAN10 | 10.28.40.0 | 255.255.248.0 | 10.28.40.1 |
| EXCLUDE ADDRESS | 10.28.40.1 |  |  |
| EXCLUDE ADDRESS | 10.28.47.255 |  |  |
| VLAN20 | 10.28.68.0 | 255.255.252.0 | 10.28.68.1 |
| EXCLUDE ADDRESS | 10.28.68.1 |  |  |
| EXCLUDE ADDRESS | 10.28.71.255 |  |  |
| VLAN30 | 10.28.82.0 | 255.255.254.0 | 10.28.82.1 |
| EXCLUDE ADDRESS | 10.28.82.1 |  |  |
| EXCLUDE ADDRESS | 10.28.83.255 |  |  |
| Distribution Router 3 |  |  |  |
| VLAN10 | 10.28.48.0 | 255.255.248.0 | 10.28.48.1 |
| EXCLUDE ADDRESS | 10.28.48.1 |  |  |
| EXCLUDE ADDRESS | 10.28.55.255 |  |  |
| VLAN20 | 10.28.72.0 | 255.255.252.0 | 10.28.72.1 |
| EXCLUDE ADDRESS | 10.28.72.1 |  |  |
| EXCLUDE ADDRESS | 10.28.75.255 |  |  |
| VLAN30 | 10.28.84.0 | 255.255.254.0 | 10.28.84.1 |
| EXCLUDE ADDRESS | 10.28.84.1 |  |  |
| EXCLUDE ADDRESS | 10.28.85.255 |  |  |

| GE2/0.10 | 10.28.32.1 | 255.255.248.0 |
| --- | --- | --- |
| GE2/0.20 | 10.28.64.1 | 255.255.252.0 |
| GE2/0.30 | 10.28.80.1 | 255.255.254.0 |

Student 2000

2048

2^11=2048 user

21 bit Network and 11 bit Host

Total ranges 10.28.32.0 /21---10.28.39.255

Subnet mask: 255.255.248.0

Teacher 1000

1024

2^10=1024

Net id 22 and Host 10 bit

10.28.64.0

Admin 500

10.28.32.0/21

10.28.0000

Core router Configuration

interface GigabitEthernet0/0

ip address 10.28.88.1 255.255.255.248

!

interface GigabitEthernet1/0

ip address 10.28.88.9 255.255.255.248

!

interface GigabitEthernet2/0

ip address 10.28.88.17 255.255.255.248

!

interface GigabitEthernet3/0

ip address 103.133.254.254 255.255.255.0

!

router ospf 1

router-id 1.2.3.0

log-adjacency-changes

redistribute bgp 65002 subnets

network 10.28.88.0 0.0.0.7 area 0

network 10.28.88.8 0.0.0.7 area 1

network 10.28.88.16 0.0.0.7 area 2

!

router bgp 65002

bgp log-neighbor-changes

no synchronization

neighbor 103.133.254.1 remote-as 65001

network 103.133.254.0 mask 255.255.255.0

redistribute ospf 1

Distribution Router 01

interface GigabitEthernet0/0

ip address 10.28.88.2 255.255.255.248

##VLAN Protocal

interface GigabitEthernet2/0.10

description vlan10_subinterface

encapsulation dot1Q 10

ip address 10.28.32.1 255.255.248.0

!

interface GigabitEthernet2/0.20

description vlan20_subinterface

encapsulation dot1Q 20

ip address 10.28.64.1 255.255.252.0

!

interface GigabitEthernet2/0.30

description vlan30_subinterface

encapsulation dot1Q 30

ip address 10.28.80.1 255.255.254.0

!

### DHCP Pool

!

ip dhcp excluded-address 10.28.32.1

ip dhcp excluded-address 10.28.39.255

ip dhcp excluded-address 10.28.64.1

ip dhcp excluded-address 10.28.67.255

ip dhcp excluded-address 10.28.80.1

ip dhcp excluded-address 10.28.81.255

!

ip dhcp pool vlan10

network 10.28.32.0 255.255.248.0

default-router 10.28.32.1

dns-server 103.133.254.2

ip dhcp pool vlan20

network 10.28.64.0 255.255.252.0

default-router 10.28.64.1

dns-server 103.133.254.2

ip dhcp pool vlan30

network 10.28.80.0 255.255.254.0

default-router 10.28.80.1

dns-server 103.133.254.2

!

OSPF Configuration

router ospf 1

router-id 1.1.1.1

log-adjacency-changes

network 10.28.88.0 0.0.0.7 area 0

network 10.28.32.0 0.0.7.255 area 0

network 10.28.64.0 0.0.3.255 area 0

network 10.28.80.0 0.0.1.255 area 0

!

Distribution 2

interface GigabitEthernet0/0

ip address 10.28.88.10 255.255.255.248

!

#VLAN

interface GigabitEthernet2/0.10

description vlan10_subinterface

encapsulation dot1Q 10

ip address 10.28.40.1 255.255.248.0

!

interface GigabitEthernet2/0.20

description vlan20_subinterface

encapsulation dot1Q 20

ip address 10.28.68.1 255.255.252.0

!

interface GigabitEthernet2/0.30

description vlan30_subinterface

encapsulation dot1Q 30

ip address 10.28.82.1 255.255.254.0

!

#DHCP

ip dhcp excluded-address 10.28.40.1

ip dhcp excluded-address 10.28.47.255

ip dhcp excluded-address 10.28.68.1

ip dhcp excluded-address 10.28.71.255

ip dhcp excluded-address 10.28.82.1

ip dhcp excluded-address 10.28.83.255

!

ip dhcp pool vlan10

network 10.28.40.0 255.255.248.0

default-router 10.28.40.1

dns-server 103.133.254.2

ip dhcp pool vlan20

network 10.28.68.0 255.255.252.0

default-router 10.28.68.1

dns-server 103.133.254.2

ip dhcp pool vlan30

network 10.28.82.0 255.255.254.0

default-router 10.28.82.1

dns-server 103.133.254.2

!

OSPF Configuration

router ospf 1

router-id 2.2.2.2

log-adjacency-changes

network 10.28.88.8 0.0.0.7 area 1

network 10.28.40.0 0.0.7.255 area 1

network 10.28.68.0 0.0.3.255 area 1

network 10.28.82.0 0.0.1.255 area 1

!

Distribution 2

interface GigabitEthernet0/0

ip address 10.28.88.18 255.255.255.248

!

#VLAN

interface GigabitEthernet2/0.10

description vlan10_subinterface

encapsulation dot1Q 10

ip address 10.28.48.1 255.255.248.0

!

interface GigabitEthernet2/0.20

description vlan20_subinterface

encapsulation dot1Q 20

ip address 10.28.72.1 255.255.252.0

!

interface GigabitEthernet2/0.30

description vlan30_subinterface

encapsulation dot1Q 30

ip address 10.28.84.1 255.255.254.0

!

###OSPF

router ospf 1

router-id 3.3.3.3

log-adjacency-changes

network 10.28.88.16 0.0.0.7 area 2

network 10.28.48.0 0.0.7.255 area 2

network 10.28.72.0 0.0.3.255 area 2

network 10.28.84.0 0.0.1.255 area 2

!